SMC IrCC

Vcomm Device Driver

Version 1.1

OEM Release Notes

Document Version: 1.02

June 6, 1997

The information contained herein is confidential, is submitted in confidence, and is proprietary information of Standard Microsystems Corporation, and shall only be used in the furtherance of the agreement of which this document forms a part, and shall not, without Standard Microsystems Corporation’s prior written approval, be reproduced or furnished to others. The information contained herein may not be disclosed to a third party without the consent of Standard Microsystems Corporation, and then, only pursuant to a Standard Microsystems Corporation approved non-disclosure agreement. Standard Microsystems Corporation assumes no liability for incidental or consequential damages arising from the use of this document contained herein, and reserves the right to update, revise, or change any information in this document without notice.

1. Summary

1.1 Introduction

1.2 Legitimate Use

1.3 Package Contents

1.4 Characteristics

1.5 Other Required Software

2. Installation and Removal

2.1 Qualification of the SMC IrCC Hardware

2.2 Preparation of the System

2.3 Installing the Driver

2.3.1 Port Installation

2.3.2 Port Configuration

2.3.3 Microsoft IrDA2.0 Installation.

2.3.4 Installing the Infrared Framer Driver for SMC IrCC.

2.3.5 Installation Results.

2.4 Optional Setting - Transceiver Selection

2.5 Removing the Driver

3. Hardware Resources

3.1 PnP BIOS Requirements

3.2 Hardware Resource Assignments

4. Use with Applications

4.1 Mapping a Network Drive

4.2 Using TP on a Network Drive

4.3 Using LapLinkÒ for Windows 95 (V7.0)

# Summary

## Introduction

The SMC IrCC Vcomm Device Driver, when combined with portions of the Microsoft Infrared Stack MSIR20, enables hardware containing an SMC IrCC (Infrared Communications Controller) to communicate with IrDA V1.1 compliant devices using wireless infrared technology for the purposes of communication, printing and networking.  This SMC IrCC Vcomm Driver is intended to be used in a system that consists of a PnP BIOS controlling the IrCC and the Microsoft Windows 95 Operating System. This Driver can also be used without such a BIOS.

This document (the OEM Release Notes) describes the system preparation, installation, use and removal of this Driver on a detailed technical level.  It is intended for software, firmware and hardware engineers who understand the architecture of Windows 95 operating system and have working knowledge of the system and its resources configuration.

## Legitimate Use

This software is provided to OEMs to be installed and included on their systems.

## Package Contents

The release diskette contains the following files:

- RelNotes.Doc - this file

- SmcSer.Vxd - Vcomm Port Driver for SMC IrCC

- SmcSer.Inf - the installation file for the SmcSer.Vxd Vcomm Port Driver

- SmcIrlap.Vxd  enhanced framer that allows fast infrared speed communications

- SmcIrlap.Inf - installation file for the SmcIrlap.Vxd

## Characteristics

- Driver Type:  Port Driver loaded by Vcomm

- Target O/S:  Windows 95

- Target Hardware:  SMC IrCC (Infrared Communications Controller)

- Network Standard: IrDA V1.1

- Speeds Supported (all IrDA 1.1 speeds):

SIR:  	2.4, 9.6, 19.2, 38.4, 57.6, 115 Kbps

FIR:  	0.576, 1.152, 4.0 Mbps (requires DMA channel)

- IR Transceivers Supported:

HP	HSDL-1100

IBM 	31T1100

Sharp	RY5FD01D

Temic	TFDS6000

## Other Required Software

Microsoft Infrared Support for Windows 95 Version 2.0 (known as MSIR20 or IrDA2.0).  This software must be obtained and licensed for redistribution from Microsoft Corporation.

In case of the retail release of Windows 95 this software must be installed externally. In case of the OSR2 and Memphis releases this software can be accessed through the Add New Hardware Wizard in Control Panel.

# Installation and Removal

This chapter explains how to install the SMC IrCC Vcomm Device Driver on the retail release of Microsoft Windows 95 Operating System.  Installation of this driver has a different format when the system is equipped with a Plug-and-Play BIOS that supports SMCF010 DevNode.  It is different yet on the OSR2 version of Microsoft Windows 96 Operating System.

For the installation to be successful one must make sure that the Infrared Hardware in the system in functioning correctly.

## Qualification of the SMC IrCC Hardware

It is strongly recommended that a suite of hardware tests is performed before the installation of the SMC IrCC Vcomm Device Driver. These tools are available from SMC as SMC IR_UTILS. Once the tools are loaded on the system run two batches of tests

MastSir ---- SlaveSir

and

MastFir ---- SlaveFir

between two nodes.

SMC IR_UTILS offer other tests, please refer to the installation notes there for more information.

## Preparation of the System

Follow ALL the steps listed below:

- Make sure that you have a Windows 95 CD ROM or a copy of the CAB files on the target system Hard Disk.

- It is absolutely necessary to remove any other IR software from the target system before attempting to install this driver.  Examples of other IR software include Microsoft Infrared Support for Windows 95 Version 3.0, Microsoft IrDA LAN, and TranxitÒ by Puma.

- It is also necessary that no other IR devices be within range for IR communications during the installation of this driver.

HINT:

To remove Microsoft Infrared Support For Windows 95 Version 2.0:

- Select: Start -- Settings -- Control Panel -- Add/Remove Software.

- Select: Microsoft Infrared Support for Windows 95 Version 3.0.

- Press: Add/Remove.  When asked to restart the computer, press YES.

HINT:

To remove TranxitÒ software:

- Select: Start -- Programs -- Accessories -- TranxitÒ.

- Select: Uninstall.

- If the Serial I/O Base chosen for the SMC IrCC indicates the COM Port that already exists in the system then remove this COM Port.

HINT:

To remove a COM Port:

- Select: Control Panel – System -- Ports (COM &LPT).

- Select: COM Port to be removed.

- Click: Remove button.

- In order to use some of the IR Applications described in this document, if it is not already installed, install Direct Cable Connection (“DCC”)

HINT:

To install Direct Cable Connection (DCC):

- Select: Start -- Settings -- Control Panel -- Add/Remove Programs -- Windows Setup -- Communications.  DCC will require DialUp Networking

.

- In order to use some of the IR Applications described in this document, if it is not already installed, install and configure File and Print Sharing For Microsoft Networks.

HINT:

To install Direct Cable Connection (DCC):

- Select: Start -- Settings -- Control Panel -- Network -- Add -- Service -- Add -- Microsoft.

- Set the desired Access Control.

- In Windows’ Explorer:

- Right-Click on at least one existing folder

- Select: Sharing,

- Select Shared As,

- Enter a Share Name,

- Set Access type to Full

- Leave Password blank or enter one if desired (consistent with the Access Control setting from File and Print Sharing for Microsoft Networks)

.

- Go to http://www.microsoft.com/windows/software/irda.htm and download the IrDA 2.0 (Infrared Driver). Follow the instructions to unzip the driver software.  Copy the unzipped files to a floppy disk or a directory on the target system (e.g., c:\msir20).

TIP:

The location of this software might change in the future, so be sure to follow the html links on Microsoft’s Web Page.

- Copy the contents of the SMC IrCC Vcomm Device Driver distribution disk onto the target system (e.g., c:\smcir20).

- If the target system is using an SMC ISA Evaluation Board (or other non-PnP hardware/BIOS combination), then you will need to know the hardware resource assignments.  The following table is provided as a convenient location to record the settings of the particular target system:

| SIR I/O | FIR I/O | IRQ | DMA |
| --- | --- | --- | --- |

HINT:

If the Target System is an SMC ISA Evaluation Board, and if the default settings of the FirSetup.Exe Utility program are used, then the table would have the following values:

| SIR I/O | FIR I/O | IRQ | DMA |
| --- | --- | --- | --- |
| 0x3e8-0x3ef | 0x240-0x247 | 5 | 1 |

- IMPORTANT: If the Target System is an SMC ISA Evaluation Board,  then it is important to run a configuration utility (e.g., SMC FirSetup.Exe) before installing this Driver (e.g., in Autoexec.Bat).

- Restart the computer after ALL the necessary preparations have been performed.

## Installing the Driver

The following instructions apply to the situation in which the referenced files were copied to the folders specified in the previous section:

- c:\msir20 and

- c:\smcir20 or a:\

If the files were placed in different folders, then substitute the actual folders containing the software in the instructions which follow.

The example installation shows the port driver and hardware being installed for the SMC IrCC with the following system resources:

- Serial I/O Base = 0x3e8 - 0x3ef 	(COM3)

- IRQ = 5

- Fast I/O Base = 0x240 - 0x248

- DMA = 1

### Port Installation

From Control Panel:

- Select: Add New Hardware

- When the Add New Hardware Wizard asks whether to search for the new hardware:

- Select the No radial button

- Select Ports (COM & LPT)

- When prompted for the type of port to install select: Have Disk

- If the SMC IrCC Vcomm Device Driver files are on the floppy disk in drive A:

then click OK,

otherwise click Browse and point to the c:\smcir20 directory with the driver files.

- When the Add New Hardware Wizard finds the SmcSer.Inf file that contains the information about the port installation it prompt with the selection of the hardware type to install:

- Choose: SMC IrCC (Fast Infrared) Hardware and Driver

- Click: Next

- These are the default installation options.

- If the default installation options match the hardware in the system

then Click: Next

otherwise Click: Edit to match the system resources. (See Configuration Section)

- Restart the system by clicking on Start – Shut Down – Restart the computer

### Port Configuration

If the configuration of the SMC IrCC (Fast Infrared) Port needs to be altered, it can be performed from

Configuration Manager.

- Select: Configuration Manager – System – Ports (COM & LPT).

- Select: SMC IrCC (Fast Infrared) Hardware and Driver (COM3).

- Click: Properties button.

- Verify that Driver tab shows SmcSer.Vxd at File version: 1.0.001.

- Select Resources tab.

- Select the configuration that matches the system resources.

CAUTION:

This installation describes and process that obtains the system resources from SmcSer.Inf file that contains the LogConfig sections that are necessary for the non PnP BIOS system.  In a PnP BIOS equipped system it is possible that the configuration resources will be organized differently than the ones based on LogConfigs .

### Microsoft IrDA2.0 Installation.

Follow the installation instructions given in the Microsoft IrDA2.0 package.  This section only highlights the areas of this installation that are different from standard and pertain to the SMC IrCC technology.

To install Microsoft IrDA2.0 infrared solution:

- Activate Setup.Exe utility in the directory c:\msir20 (or the directory that contains this software).

- Choose: (Standard infrared devices) – Built-in Infrared port on laptop or desktop; this is the default selection.

- Click: Next

- When Add Infrared Device Wizard offers the port choices for the installation:

- Choose: SMC IrCC (Fast Infrared) Hardware and Driver (COM3).

- Complete the Microsoft IrDA2.0 installation according to the instructions from Microsoft.

### Installing the Infrared Framer Driver for SMC IrCC.

This is the last step of the installation. It must be done AFTER the installation of Microsoft IrDA2.0 software. This will install the Infrared Framer Driver for SMC IrCC as well as update the Windows Registry with the keys needed by the framer.

From the Windows Explorer:

- Select: c:\smcir20 or a:\  ; depending where the SMC IrCC software resides.

- Select SmcIrlap.Inf. This is the installation file for the Framer. This file will run “silently” that is there will not be a user dialog regarding the resources.

- Right-click brings the menu for this file.

- Select: Install

After the SmcIrlap.Inf completes the installation of the Framer, it is a good time to verify the Registry contents.  From Windows Explorer:

- Select Windows 95 home directory

- Select: Regedit.Exe

- Point to: HKEY_LOCAL_MACHINE, Enum\Infrared\Framer:

Make sure that the values of the Framer key match the list above.

Finally Restart the System.

After the System starts again it is ready to communicate over Infrared medium. Simply point the infrared transceiver toward an active infrared station and the two stations should connect and display each other’s name in the Infrared Monitor.

### Installation Results.

If the installation was performed properly, the following is the state of various files and resources:

- (Windows Home Dir)\System\SmcSer.Vxd

- (Windows Home Dir)\System\SmcIrlap.Vxd

- (Windows Home Dir)\Inf\SmcSer.Inf

- Control Panel contains Infrared Monitor

## Optional Setting - Transceiver Selection

For the systems that do not need the SmcSer.Vxd Driver to determine the transceiver type, but want to specify the transceiver type themselves there exists an option to do this:

Using the Regedit.Exe - Registry editor, point to the key:

HKEY_LOCAL_MACHINE, Enum\Infrared\Framer

and insert the BINARY value:

TransceiverType.

The values for this key are:

0 - Automatic selection

1 - IBM/Temic type

2 - H-P/Sharp type

## Removing the Driver

To remove the infrared software, in Control Panel:

- Select: Start -- Settings -- Control Panel -- Add/Remove Software.

- Select: Infrared Software for Windows 95 Version 2.0

- Click: Add/Remove.

- When asked to restart the computer, click: YES.

After the infrared software, the infrared port will have to be removed:

- Select: Control Panel – System -- Ports (COM &LPT).

- Select: SMC IrCC (Fast Infrared) Hardware and Driver (COM3).

- Click: Remove button.

After reboot, erase the following disk files:

- (Windows Home Dir)\system\SmcSer.Vxd

- (Windows Home Dir)\system\SmcIrlap.Vxd

- (Windows Home Dir)\inf\SmcSer.Inf

- (Windows Home Dir)\inf\oem*.Inf

# Hardware Resources

## PnP BIOS Requirements

IMPORTANT:

A PnP BIOS is an “all or nothing” thing:  if a BIOS returns a DevNode for an SMC IrCC, then it must implement every feature and function of that DevNode properly, or else (in general)  it will not be possible to properly load and execute any PnP Driver.

It is the responsibility of the BIOS to perform any configuration activities that cannot be done from the IrCC run-time registers.  This includes any of the Configuration Registers in any of the devices that contain the IrCC, as well as the registers in the 8051 address space in the FDC37C957FR.  Examples of these functions include pin function/polarity/direction assignments, turn-around time, i/o mux selection, etc.  The IR Options register must be set to place the IrCC into one of the IrDA SIR operating modes if a Compatibility ID of PNP0511 is to be returned.  All IRQ’s and DMA should be masked (which they are after IrCC reset is finished).

The Device ID for the SMC IrCC is SMCF010.  Note that if a  Device ID (as opposed to a Computability ID) of PNP0511 is used for the DevNode, then this will indicate that the DevNode consists of SIR-only hardware, and (in general) it will not be possible to install any PnP FIR Device Driver on the hardware.  If this is really desired, then the SCE portion of the IrCC should be disabled, and no DMA channel should be assigned.  It is assumed that, for the purposes of a document describing an FIR Driver, such an arrangement is not desired, so no more will be said about it.

WARNING:

If the PnP ID is PNP0511 and/or the DevNode does not contain all of the resources needed by a PnP FIR Driver, then the best that can be hoped for is that the OS will refuse to even install the Driver.  If a determined end-user is somehow able to convince the OS to install the Driver, then the next best situation to be hoped for is that the Driver will refuse to load and execute.  If, somehow, the Driver ever does execute, then the best that can be hoped for is that the system will blue-screen and halt before any serious damage is done.  The BIOS author is being warned here that there is no imaginable situation in which anything good could ever come of a situation like this.

When the BIOS returns Resource TAG’s for the IrCC, it is important that the first I/O TAG is for the SIR, and the second I/O TAG is for the FIR;  any additional I/O TAG’s will be ignored.  The order of the DMA and IRQ TAG’s is unimportant since there will only be one of each.  Note that this arrangement is also compatible with the requirements of the SMC IR NDIS 4.2 Miniport Driver, Puma Technology  FIR device drivers, as well as the various Microsoft SIR device drivers for PNP0511 devices.

It is required that two (2) I/O and one (1) IRQ TAG must always be present.  Each of the I/O blocks must contain 8 contiguous ports aligned on an 8-port boundary;  see the various SMC data sheets for other restrictions that vary by device type.  The IRQ is assumed to be legacy ISA type and sharing is discouraged since not all Drivers will function properly if the IRQ is shared.  However, if the IRQ is shared, then this Driver requires that the sharing mechanism be that specified by Microsoft for ISA IRQ sharing on Windows 95.  It is expected, but this driver does not require, that:

- SirBase choices will be 0x3f8/0x2f8/0x3e8/0x2e8 with a 16-bit I/O decode.

- A choice of at least 4 FirBase values with 16-bit I/O decode.

- IRQ choices will be 3/4/10/11.

- If DMA is used, at least 3 choices of DMA channel assignment.

The above suggestions are typical of standard practice in modern PC designs, and are suggestions only.  The actual requirements for this Driver are:

- Each I/O Base address must be aligned on an 8-port boundary, [0x100,0xff8].

- The IRQ must be Î [0,15] and != 2.

- The DMA (if any) must be Î [0,7] and != 4.

There are three (3) possible DMA configurations, each possessing different performance and IrDA speed capabilities.  An 8-bit DMA channel is a “Good configuration” (DF value = 0) and results in the highest performance with all IrDA speeds being available.  A 16-bit DMA channel is an “Acceptable configuration” (DF Value = 1) with all IrDA speeds being available, but having slightly reduced performance when compared with an 8-bit DMA channel.  No DMA channel is a “Sub-optimal configuration” (DF value = 2) in which only SIR speeds will be available from this Driver.  (While it is technically feasible to offer the 576 Kbps FIR speed without a DMA channel, it is not expected that many Driver authors will bother to do so.)

If a DMA channel is available in the configuration, the Driver will assume Legacy channel width (i.e., DMA 0-3 are 8-bit channels, and DMA 5-7 are 16-bit channels);  this restriction is imposed by the NDIS environment.  Timing is assumed to be Legacy, and the Driver will use the channel in Demand Transfer mode.

If hot docking/undocking is supported in the system, and if this device is affected by the event either through port replication or transceiver type switching, then it is the responsibility of the BIOS to notify the OS in such a way as to halt this Driver before the event, and to re-initialize this Driver after the event.

## Hardware Resource Assignments

This Driver makes use of the PnP facilities available to Port Device Drivers on Windows 95 in order to obtain its hardware resource assignments.  A Resources property sheet exists in Device Manager through which resource assignments can be viewed and changed.  For the case of ISA Evaluation Boards in which a PnP BIOS is not controlling the IrCC, the choices of possible resource assignments are obtained from the LogConfig sections in the SmcSer.Inf file.  See this section of the file and the comments for more information.

HINT:

For the case of ISA Evaluation Boards, a configuration utility (e.g., SMC FirSetup.Exe) must be executed before the starting the GUI in the OS (e.g., in AutoExec.Bat).

# Use with Applications

## Mapping a Network Drive

Briefly, DCC (Direct Cable Connection) is an applet that ships with Windows 95 and permits the connection of 2 machines in a peer-to-peer network configuration.  One machine is designated as the HOST, and is the machine whose resources become available for use by the GUEST.

After deciding which machine is to be the Host, use Explorer (Start--Programs--Accessories--Windows Explorer) to make a subdirectory sharable by selecting the folder, then File--Properties--Sharing.  Select Shared As, define a Share Name, and set the desired Access Type.  Then execute DCC (Start--Programs--Accessories--Direct Cable Connection) on the Host machine.  Note that DCC is not installed as part of the standard Windows 95 Setup, so it might need to be installed if it has not been already.  Define the machine as a Host, click Next, define the port as “Serial cable on COM4”, click Next, select Use Password protection (if desired, and define the password), click Finish.  The Host machine will then begin waiting for a Guest machine on COM4, which is the default IR COM port.

On the Guest machine, start DCC and define the machine as a Guest on COM4.  Once the machines have established the connection, the Guest machine will look for, and display, any shared folders.  Go to Explorer on the Guest machine and select Tools -- Map Network Drive.  Select the desired drive letter (e.g., E:), and the Path to connect to (e.g., \\name\folder, where “name” is the name of the host machine, as defined on the Identification tab in Infrared Monitor, and “folder” is the “Shared As” name for the folder as defined on the Sharing tab on the host machine).  Click OK.  The shared folder should then display on the left pane of the Explorer display, and can be accessed like any other drive.  This includes the ability to drag-and-drop files between folders and drives.

## Using TP on a Network Drive

TP is a simple DOS command-line utility program that can be useful for exercising network communications links.  For the purpose of exercising the IR link, the program can be used to write data to the network drive, then read it back and compare it to the original, and display performance and error statistics.  Typing TP <enter> with no command-line switches will display a list of available options.  An example command line is as follows:  tp e:testfile I=10 60000 <enter>.  This will create a file called “testfile” on the network drive e: with a data size of 60,000 bytes.  The program will write and then read the file 10 times, and then display the results.   For more accurate performance measurements, the “K” option should be used to kill the display during the transfer.

## Using LapLinkÒ for Windows 95 (V7.0)

Install and start LapLinkÒ on both test machines.  Configure e.g., COM4 as a Wireless Port and connect the machines.  A pair of windows similar to those in  Explorer will display that permits drag-and-drop of files, etc.  Consult the LapLinkÒ documentation for more information.