SMC IrCC NDIS 4.0

Miniport Device Driver

Version 2.1

OEM Release Notes

July 29, 1997

The information contained herein is confidential, is submitted in confidence, and is proprietary information of Standard Microsystems Corporation, and shall only be used in the furtherance of the agreement of which this document forms a part, and shall not, without Standard Microsystems Corporation’s prior written approval, be reproduced or furnished to others. The information contained herein may not be disclosed to a third party without the consent of Standard Microsystems Corporation, and then, only pursuant to a Standard Microsystems Corporation approved non-disclosure agreement. Standard Microsystems Corporation assumes no liability for incidental or consequential damages arising from the use of this document contained herein, and reserves the right to update, revise, or change any information in this document without notice.

Table of Contents

Table of Contents

Chapter 1 -- Summary

1.1  Introduction

1.2  Legitimate Use

1.3  Package Contents

1.4  Characteristics

1.5  Other Required Software

Chapter 2 -- Installation and Removal

2.1  Before Installing the Driver

2.2  Installing the Driver

2.3  Installing IrDA LAN Software

2.4  Removing the Driver

Chapter 3 -- Advanced Properties

3.1  Infrared Transceiver Type

3.2  Minimum Turn-Around Time

3.3  Speed Enable

Chapter 4 -- Hardware Resources

4.1  PnP BIOS Requirements

4.2  Hardware Resource Assignments

4.3  Yielding Hardware Resources

Chapter 5 -- Use with Applications

5.1  Mapping a Network Drive

5.2  Using TP on a Network Drive

5.3  LaplinkÒ for Windows 95 (V7.0)

5.4  Network Access Point Devices

5.5  Microsoft IrXfer

Chapter 6 -- Installation/Removal Issues

6.1  Installation Issues

6.2  Removal Issues

6.3  Alternative Installation Methods

6.3.1  Modifying Infrared.Inf

6.3.2  Network Add-Adapter

Chapter 7 -- Error Codes

Chapter 8 -- Known Bugs

Chapter 1 -- Summary

1.1  Introduction

The SMC IrCC NDIS 4.0 Miniport Device Driver, when combined with portions of the Microsoft IR30DDK, enables hardware containing an SMC IrCC (Infrared Communications Controller) to communicate with IrDA V1.1 compliant devices using wireless infrared technology for the purposes of communication, printing and networking.  Although the ultimate Target System will consist of a PnP BIOS controlling the IrCC and the Microsoft “Memphis” Operating System, this Driver can also be used without such a BIOS and with the shipping versions of Windows 95 instead of “Memphis”.

This document (the OEM Release Notes) describes the system preparation, installation, use and removal of this Driver on a detailed technical level.  It is intended for software, firmware and hardware engineers with a thorough working knowledge of these areas, and has not been written for end-users.

1.2  Legitimate Use

This software is being provided FOR TESTING PURPOSES ONLY!

No license for any other purpose is either granted or implied.

1.3  Package Contents

RelNotes.Doc -- this file

SmcIrda.Sys -- the actual driver

SmcIrda.Inf -- the installation file

1.4  Characteristics

Driver Type:  NDIS 4.0 Miniport

Target O/S:  Windows 95, Memphis

Target Hardware:  SMC IrCC (Infrared Communications Controller)

Network Standard: IrDA V1.1

Speeds Supported (all IrDA 1.1 speeds):

SIR:  	2.4, 9.6, 19.2, 38.4, 57.6, 115 Kbps

FIR:  	0.576, 1.152, 4.0 Mbps (requires DMA)

IR Transceivers Supported:

HP	HSDL-1100/2100/2300

IBM 	31T1100/1200

Sharp	RY5FD01D

Temic	TFDS6000

1.5  Other Required Software

Microsoft Infrared Support for Windows 95 Version 3.0

Chapter 2 -- Installation and Removal

NOTE	   It is STRONGLY recommended that the prospective user obtain a copy of the SMC IR_UTILS disk, and run the MastSir/SlaveSir and MastFir/SlaveFir programs to determine proper hardware operation BEFORE attempting to use this, or any other, device driver software.

EXTREMELY IMPORTANT:

- It is absolutely necessary to remove any other IR software from the target system before attempting to install this driver.  Examples of other IR software include Microsoft Infrared Support for Windows 95 Version 2.0, Microsoft IrDA LAN, and TranxitÒ by Puma.

- It is also necessary that no other IR devices be within range for IR communications during the installation of this driver.

HINT	To remove Microsoft IrDA LAN Software, first be sure that the JetEyeÒ, NetBeamIR (or equivalent) is out of range of the SmcIrda station.  Use Start -- Settings -- Control Panel -- Network.  Select the Microsoft IrDA LAN Driver, and press Remove.  Press OK to close the Network dialog.  More files will copy and more Version Conflict dialogs will appear;  press YES to each one in order to keep all of the newer files.  When asked to restart the computer, press NO.

HINT   To remove Microsoft Infrared Support For Windows 95 Version 2.0, select Start -- Settings -- Control Panel -- Add/Remove Software.  Select Microsoft Infrared Support for Windows 95 Version 2.0.  Press Add/Remove.  When asked to restart the computer, press YES.

HINT   To remove TranxitÒ software, select Start -- Programs -- Accessories -- TranxitÒ.  Select Uninstall.

2.1  Before Installing the Driver

You will need a Windows 95 CD ROM, or a copy of the CAB files on the target system.

In order to use some of the IR Applications described in this document, if it is not already installed, install Direct Cable Connection (“DCC”) using Start -- Settings -- Control Panel -- Add/Remove Programs -- Windows Setup -- Communications.  DCC will require DialUp Networking.

In order to use some of the IR Applications described in this document, if it is not already installed, install and configure File and Print Sharing For Microsoft Networks using Start -- Settings -- Control Panel -- Network -- Add -- Service -- Add -- Microsoft.  Set the desired Access Control.  In Start -- Programs -- Windows Explorer, right-click on at least 1 folder, select Sharing, select Shared As, enter a Share Name, set Access type to Full, and either leave Password blank or enter one if desired (consistent with the Access Control setting from File and Print Sharing for Microsoft Networks).

Go to http://www.microsoft.com/windows/software/irda.htm and download the Windows95 Fast Infrared 3.0 DDK (published:  January 16, 1997).  Follow the instructions to unzip and install the DDK.  Copy the IR30\Retail directory onto the target system (e.g., c:\ir30).  If the target has Memphis, then IR30 is already on the system.

TIP   The location of this software (like everything on the Web)   might change in the future, so be sure to follow the html links on Microsoft’s Web Page.

Copy the SmcIrda.* files from this package onto the Target System (e.g., c:\smcirda).

If the target system is using an SMC ISA Evaluation Board (or other non-PnP hardware/BIOS combination), then you will need to know the hardware resource assignments.  The following table is provided as a convenient location to record the settings of the particular target system:

| SIR I/O | FIR I/O | IRQ | DMA |
| --- | --- | --- | --- |

HINT   If the Target System is an SMC ISA Evaluation Board, and if the default settings of the FirSetup.Exe Utility program are used, then the correct values for the previous Table would be:

0x3e8-0x3ef  /  0x240-0x247  /  5  /  1

IMPORTANT  If the Target System is an SMC ISA Evaluation Board,  then it is important to run a configuration utility (e.g., SMC FirSetup.Exe) before installing this Driver (e.g., in Autoexec.Bat).

2.2  Installing the Driver

NOTE   The following instructions apply to the situation in which the referenced files were copied to the folders specified in the previous section (i.e., c:\ir30 and c:\smcirda).  If the files were placed in different folders, then substitute the actual folders containing the software in the instructions which follow.

The following instructions apply to the Retail version of the Microsoft Windows 95 Operating System and to SMC ISA Evaluation Boards, which are not controlled by a PnP BIOS;  the procedure for other hardware and software environments is similar, but the details will vary slightly.  In particular, for Memphis, IR30 is in the CAB files, so use Add New Hardware -- Have Disk, instead of Start -- Run -- Setup.Exe -- Have Disk.  Also, on Memphis, a Resources sheet (correctly) appears during the installation if the hardware is not controlled by a PnP BIOS;  be sure to set the desired values when it does.

Start Windows 95 on the Target System.

Use Start--Run--[browse]--c:\ir30\setup.exe, press OK.

When asked if you wish to install IR30, press YES.

When the Add Infrared Device Wizard displays, press NEXT.

(The Wizard will build a driver database.)

Press Have Disk... [Browse...] c:\smcirda, press OK.

When the Wizard shows a Model of SMC IrCC, press OK.

(The Wizard will copy files and make another driver database.)

When the Wizard asks for Windows 95 CD-ROM, press OK.

Enter the path to the Windows 95 CD-ROM (or CAB files).

Press OK.

When the Version Conflict dialog asks if you want to keep any newer files, press YES.

The Wizard will offer of choice of Use defaults (recommended) or Change Settings radio buttons.  The default values are:

Transceiver Type = AUTO

Min. TurnAround Time = 1.0 mS

IR COM port = COM4

IR LPT port = LPT3

If it is desired to change any of these values, then select Change Settings and press NEXT to step through each choice;  otherwise, leave Use defaults (recommended) selected and press NEXT.  Press Finish.

(The Wizard will copy more files, and then will Find New Hardware twice [IR COM and IR LPT]).

When asked to reboot, press OK.

After first reboot, select Start--Settings--Control Panel--Network.

Press Add -- Protocol -- Add -- Microsoft -- Fast Infrared Protocol -- OK.

HINT   For the case of SMC ISA Evaluation Boards that are not controlled by a PnP BIOS, the Table of Resource Assignments in Section 2.1 should have been filled, and the values in that table should be used in the next step.

Select SMC IrCC -- Properties -- Resources.  Set values.

Press OK.

Press OK to close Network.  When asked to restart, press YES.

After second reboot, the IR software should be fully installed and operational.

2.3  Installing IrDA LAN Software

When an access point device (e.g., ESI-9910 JetEye or HP NetBeamIR) is placed within range of the SmcIrda station, it should be automatically detected and the required software will attempt to install itself.  It will be necessary to have the Windows 95 CD-ROM (or copies of the CAB files on the target system).  As above, whenever any Version Conflict dialog(s) asks if you want to keep any newer files, press YES.

After the initial installation, NetWare servers will not be visible on the network because the IPX/SPX protocol will be bound to the DialUp Adapter.  Select Start -- Settings --  Control Panel -- Network.  Select the DialUp Adapter -- Properties -- Bindings.  Disable the check box for IPX/SPX, and press OK.  Press OK again to close the Network dialog.  More files will be copied, and more Version Conflict dialogs will appear;  answer YES to each one in order to keep all of the newer files.  When asked to restart the computer, press YES.

After reboot, the IrDA LAN software should be fully installed and operational.

2.4  Removing the Driver

EXTREMELY IMPORTANT

- Before removing this Driver, be sure that no IR Devices are presently in range and communicating with the Target System.

- If IrDA LAN Software has been installed, then remove that software (as described next) BEFORE removing the Driver.

Before removing the driver, be sure to terminate any IR communications with any other IR devices.  If the Driver was used with a Network Access Point Device (e.g., ESI-9910 JetEyeÒ Net Plus, HP NetBeamIR or equivalent), then IrDA LAN software should have been installed (automatically).  If this is the case, then it should be removed (manually) before the IR30 software is removed (manually).

Before doing this, be sure that the JetEyeÒ, NetBeamIR (or equivalent) is out of range of the SmcIrda station.  Use Start -- Settings -- Control Panel -- Network.  Select the Microsoft IrDA LAN Driver, and press Remove.  Press OK to close the Network dialog.  More files will copy and more Version Conflict dialogs will appear;  press YES to each one in order to keep all of the newer files.  When asked to restart the computer, press NO.

To remove IR30, use Start -- Settings -- Control Panel -- Add/Remove Software.  Select Infrared Software for Windows 95 Version 3.0 and press Add/Remove.  When asked to restart the computer, press YES.

After reboot, erase the following three (3) disk files:

%SYSTEM%\system\smcirda.sys

%SYSTEM%\inf\smcirda.inf

%SYSTEM%\inf\oem*.inf [will be oem0.inf if no others existed before install]

Chapter 3 -- Advanced Properties

By selecting the SMC IrCC -- Properties -- Advanced tab, all of the registry key values can be inspected and manipulated.  The nature of NDIS requires that the system will need to be restarted before any new values will take effect.  In most situations, the default values for these keys in SmcIrda.Inf will be correct.

3.1 Infrared Transceiver Type

The “Infrared Transceiver Type” has a default value of AUTO, which will cause the Driver to automatically detect the transceiver type each time the Driver is Initialized.  As part of the automatic detection process, a single spurious IR pulse of about 2 microseconds duration will be transmitted if the actual Transceiver type is HP or SHARP.  Should this spurious transmission be unacceptable in a particular application, which is not usually the case, then this key should be used to inform the Driver of the particular Transceiver Type that is connected, which will also disable the detection algorithm.

3.2  Minimum Turn-Around Time

The “Min. Turn-Around Time” is an IrDA IrLAP negotiated parameter with a default value of 1 mS.  This key should be set to a value that is greater than the typical IRQ latency for the particular target system;  it is perfectly acceptable for the peak (or maximum) IRQ latency to be well above the value set for this key.  Usually, the default value of 1 mS is the best choice, but a higher value might be required if some other Driver(s) in the system are often leaving IRQ’s disabled for an extended period of time.

3.3  Speed Enable

The “Speed Enable” key is not nearly as user-friendly as either of the others, but does provide a mechanism by which each of the possible IrDA speeds can be selectively disabled should such a need arise.  As described in the comments in SmcIrda.Inf, the value of this key is a hexadecimal representation of a binary number in which (i) each binary bit position corresponds to a particular IrDA speed, (ii) setting a bit to a logic 1 enables the corresponding speed, and (iii) clearing a bit to a logic 0 disables the corresponding speed.  Under normal circumstances, the default value (which enables all IrDA speeds) should be used for this key.   For readers familiar with IrLAP, the value of this key is identical to the IrLAP negotiation parameter (PI = 0x01) for Baud Rate.

Note that the Driver will automatically disable all FIR speeds (i.e., all speeds > 115.2 Kbps) if there is no DMA channel assigned to this device at the time when the driver initializes, independent of the value of this key.  Therefore,  it is not necessary to keep changing this key value in order to tolerate the situation in which sometimes there is a DMA channel available and sometimes there is not;  the Driver will adapt automatically.

Chapter 4 -- Hardware Resources

4.1  PnP BIOS Requirements

IMPORTANT   A PnP BIOS is an “all or nothing” thing:  if a BIOS returns a DevNode for an SMC IrCC, then it must implement every feature and function of that DevNode properly, or else (in general)  it will not be possible to properly load and execute any PnP Driver.

It is the responsibility of the BIOS to perform any configuration activities that cannot be done from the IrCC run-time registers.  This includes any of the Configuration Registers in any of the devices that contain the IrCC, as well as the registers in the 8051 address space in the FDC37C957FR.  Examples of these functions include pin function/polarity/direction assignments, turn-around time, i/o mux selection, etc.  The IR Options register must be set to place the IrCC into one of the IrDA SIR operating modes if a Compatibility ID of PNP0511 is to be returned.  All IRQ’s and DMA should be masked (which they are after IrCC reset is finished).

The Device ID for the SMC IrCC is SMCF010.  Note that if a  Device ID (as opposed to a Compatability ID) of PNP0511 is used for the DevNode, then this will indicate that the DevNode consists of SIR-only hardware, and (in general) it will not be possible to install any PnP FIR Device Driver on the hardware.  If this is really desired, then the SCE portion of the IrCC should be disabled, and no DMA channel should be assigned.  It is assumed that, for the purposes of a document describing an FIR Driver, such an arrangement is not desired, so no more will be said about it.

WARNING   If the PnP ID is PNP0511 and/or the DevNode does not contain all of the resources needed by a PnP FIR Driver, then the best that can be hoped for is that the OS will refuse to even install the Driver.  If a determined end-user is somehow able to convince the OS to install the Driver, then the next best situation to be hoped for is that the Driver will refuse to load and execute.  If, somehow, the Driver ever does execute, then the best that can be hoped for is that the system will blue-screen and halt before any serious damage is done.  The BIOS author is being warned here that there is no imaginable situation in which anything good could ever come of a situation like this.

When the BIOS returns Resource TAG’s for the IrCC, it is important that the first I/O TAG is for the SIR (i.e., the ACE or UART portion of the IrCC), and the second I/O TAG is for the FIR (i.e., the SCE portion of the IrCC).  Although there will usually only be two (2) I/O TAG’s, additional I/O TAG’s, if any, will be ignored.  The first IRQ TAG is the one that will be used by the Driver.  There will usually only be only a single IRQ TAG, but additional IRQ TAG’s, if any, will be ignored.  If there are any DMA TAG’s, then the first DMA TAG is the one that will be used by the Driver, and additional DMA TAG’s, if any, will be ignored.  The order of the IRQ and DMA TAG’s with respect to the I/O TAG’s and each other is not important.  Note that this arrangement is also compatible with the requirements of the SMC IR_VCOMM Driver, Puma Technology  FIR device drivers, as well as the various Microsoft SIR device drivers for PNP0511 devices.

It is required that two (2) I/O and one (1) IRQ TAG must always be present.  Each of the I/O blocks must contain 8 contiguous ports aligned on an 8-port boundary;  see the various SMC data sheets for other restrictions that vary by device type.  The IRQ is assumed to be legacy ISA type and sharing is discouraged since not all Drivers will function properly if the IRQ is shared.  However, if the IRQ is shared, then this Driver requires that the sharing mechanism be that specified by Microsoft for ISA IRQ sharing on Windows 95.  It is expected, but this driver does not require, that:

- SirBase choices will be 0x3f8/0x2f8/0x3e8/0x2e8 with a 16-bit I/O decode.

- A choice of at least 4 FirBase values with 16-bit I/O decode.

- IRQ choices will be 3/4/10/11.

- If DMA is used, at least 3 choices of DMA channel assignment.

The above suggestions are typical of standard practice in modern PC designs, and are suggestions only.  The actual requirements for this Driver are:

- Each I/O Base address must be aligned on an 8-port boundary, [0x100,0xff8].

- The IRQ must be Î [0,15] and != 2.

- The DMA (if any) must be Î [0,7] and != 4.

There are three (3) possible DMA configurations, each possessing different performance and IrDA speed capabilities.  An 8-bit DMA channel is a “Good configuration” (DF value = 0) and results in the highest performance with all IrDA speeds being available.  A 16-bit DMA channel is an “Acceptable configuration” (DF Value = 1) with all IrDA speeds being available, but having slightly reduced performance when compared with an 8-bit DMA channel.  No DMA channel is a “Sub-optimal configuration” (DF value = 2) in which only SIR speeds will be available from this Driver.  (While it is technically feasible to offer the 576 Kbps FIR speed without a DMA channel, it is not expected that many Driver authors will bother to do so.)

If a DMA channel is available in the configuration, the Driver will assume Legacy channel width (i.e., DMA 0-3 are 8-bit channels, and DMA 5-7 are 16-bit channels);  this restriction is imposed by the NDIS environment.  Timing is assumed to be Legacy, and the Driver will use the channel in Demand Transfer mode.

If hot docking/undocking is supported in the system, and if this device is affected by the event either through port replication or transceiver type switching, then it is the responsibility of the BIOS to notify the OS in such a way as to halt this Driver before the event, and to re-initialize this Driver after the event.

4.2  Hardware Resource Assignments

This Driver makes use of the PnP facilities available to NDIS Miniport Device Drivers on Windows 95 in order to obtain its hardware resource assignments.  A Resources property sheet exists in both Device Manager and Network through which resource assignments can be viewed and changed.  For the case of ISA Evaluation Boards in which a PnP BIOS is not controlling the IrCC, the choices of possible resource assignments are obtained from the LogConfig sections in the SmcIrda.Inf file.  See this section of the file and the comments for more information.

HINT   For the case of ISA Evaluation Boards, a configuration utility (e.g., SMC FirSetup.Exe) must be executed before starting the GUI in the OS (e.g., in AutoExec.Bat).

4.3  Yielding Hardware Resources

This Driver assumes that the IrCC is connected directly to an Infrared Transceiver, and that the SIR portion of the device cannot be used as a standard COM port.  As a result, the YIELDING API is not supported by this driver (i.e., the OID’s to release and reacquire hardware resources).  For more information, consult the ReadMe.Doc file contained in the IR30DDK.

Chapter 5 -- Use with Applications

5.1  Mapping a Network Drive

Briefly, DCC (Direct Cable Connection) is an applet that ships with Windows 95 and permits the connection of 2 machines in a peer-to-peer network configuration.  One machine is designated as the HOST, and is the machine whose resources become available for use by the GUEST.

After deciding which machine is to be the Host, use Explorer (Start--Programs--Accessories--Windows Explorer) to make a subdirectory sharable by selecting the folder, then File--Properties--Sharing.  Select Shared As, define a Share Name, and set the desired Access Type.  Then execute DCC (Start--Programs--Accessories--Direct Cable Connection) on the Host machine.  Note that DCC is not installed as part of the standard Windows 95 Setup, so it might need to be installed if it has not been already.  Define the machine as a Host, click Next, define the port as “Serial cable on COM4”, click Next, select Use Password protection (if desired, and define the password), click Finish.  The Host machine will then begin waiting for a Guest machine on COM4, which is the default IR COM port.

On the Guest machine, start DCC and define the machine as a Guest on COM4.  Once the machines have established the connection, the Guest machine will look for, and display, any shared folders.  Go to Explorer on the Guest machine and select Tools -- Map Network Drive.  Select the desired drive letter (e.g., E:), and the Path to connect to (e.g., \\name\folder, where “name” is the name of the host machine, as defined on the Identification tab in Infrared Monitor, and “folder” is the “Shared As” name for the folder as defined on the Sharing tab on the host machine).  Click OK.  The shared folder should then display on the left pane of the Explorer display, and can be accessed like any other drive.  This includes the ability to drag-and-drop files between folders and drives.

5.2  Using TP on a Network Drive

TP is a simple DOS command-line utility program that can be useful for exercising network communications links.  For the purpose of exercising the IR link, the program can be used to write data to the network drive, then read it back and compare it to the original, and display performance and error statistics.  Typing TP <enter> with no command-line switches will display a list of available options.  An example command line is as follows:  tp e:testfile I=10 60000 <enter>.  This will create a file called “testfile” on the network drive e: with a data size of 60,000 bytes.  The program will write and then read the file 10 times, and then display the results.   For more accurate performance measurements, the “K” option should be used to kill the display during the transfer.

5.3  LapLinkÒ for Windows 95(V7.0) Ò

Install and start LapLinkÒ (V7.0 or later) on both test machines.  Configure e.g., COM4 as a Wireless Port and connect the machines.  A pair of windows similar to those in  Explorer will display that permits drag-and-drop of files, etc.  Consult the LapLinkÒ documentation for more information.

5.4 Network Access Point Devices

When an Extended SystemsÒ JetEyeÒ Net Plus, HP NetBeamIR, or other Network Access Point Device (i.e., NAPD) is linked with the NDIS station, and if Plug and Play software installation is enabled in IR Monitor, Windows95 will automatically install the Microsoft IrDA LAN and other required software, assuming that it has not already been installed.  Be sure to follow all of the manufacturer’s  instructions and troubleshooting information, especially about removing the IPX/SPX binding to the Dial-Up Adapter.

Operation has been confirmed at 4 Mbps FIR and 115 Kbps SIR.  If a problem is encountered with a JetEye not linking at 4 Mbps, it could be because of old firmware in the JetEye not supporting that speed.  Revised firmware is available on ESI’s website (http://www.extendsys.com).

Once the LAN connection is established, it is possible to run TP on a mapped network drive, similar to the case with DCC.  To do this, go into Explorer and select Network Neighborhood.  Select a server and open it;  it might be necessary to enter a user name and/or password depending on the network security arrangement.  Once a server has been opened, use Tools -- Map Network Drive to perform the drive mapping.  Then execute TP as described previously.

5.5  Microsoft IrXfer

To transfer files using IrXfer, select a file (e.g., in Explorer), and right-click on it.  In the pop-up, select Send To -- Infrared Recipient.  Files will be received by default in the c:\My Received Files directory.

Chapter 6 -- Installation/Removal Issues

6.1  Installation Issues

A review of the Installation procedure earlier in this document will reveal a number of undesirable behaviors.  Largely, these are the result of the combination of (i) using the “Have Disk” installation in the Wizard and (ii) installing on non-PnP hardware.  In particular:

1.  An OEM*.INF file is created, which should not be, and it is not erased on removal.  The file must be manually erased, if desired.

2.  A resources sheet is not displayed for non-PnP devices (e.g., SMC ISA Evaluation Boards) so no resources are assigned to the IrCC during installation.  The resources must be manually assigned during a subsequent session.

3.  The Fast IR Transport is not installed and bound to the IrCC.  The protocol must be manually added during a subsequent session.

4.  When IrDA LAN software installs, it leaves IPX/SPX bound to the DialUp adapter.  The binding must be manually disabled and the system restarted if it is desired for IrDA LAN to access any NetWare hosts on the network.

6.2  Removal Issues

1.  The IrDA LAN instance is not removed from the class\net portion of the Registry when IR30 is removed.  A workaround is to either (i) manually remove IrDA LAN before removing IR30 (as was advocated earlier in this document), or (ii) manually delete the instance in class\net with RegEdit after IR30 is removed (obviously not a suitable method for an end-user).

2.  The SmcIrda.Inf REMOVE script is not executed on remove, so the SmcIrda.* files are not erased.  These files must be manually erased during a subsequent session, if desired.

6.3  Alternative Installation Methods

6.3.1  Modifying Infrared.Inf

It is possible to add SmcIrda to the Infrared.Inf file in IR30, which is a file that is and remains the property of Microsoft Corporation.  If this is done, then the SMC device will appear in the Wizard without having to press the Have Disk button during installation.  In addition, there will be no OEM*.INF created during installation, and both SmcIrda.* files will be erased on remove.  However, a Resources sheet will still not be displayed during installation, and instead of not having the FastIR install, it will install with multiple bindings.  Because of this, on restart, it is necessary to go into Network, select the DialUp Adapter Bindings and disable FastIR, then select the IrCC and set the Resource values and restart.  Also, the situation with IrDA LAN remains unchanged.

To do this, it is necessary to make three (3) additions to Infrared.Inf:

1.  In the [INF_Copy] section, add smcirda.inf to the list of inf’s to copy;  the same list is used for delete on removal.

2.  In the [Driver_Delete] section, add smcirda.sys to the list of drivers to delete on removal.

3.  In the [Generic_AddReg] section, add a KnownDevices line for *SMCF010:

HKLM,Enum\INFRARED\KnownDevices,*SMCF010,0,”*SMCF010”

6.3.2  Network Add-Adapter

As documented in the comments in SmcIrda.Inf, the ir_ndi.dll line in the INF cannot be executed by shipping versions of Windows 95.  If this line is removed, then the INF will execute properly, both on Install and Remove, but the Driver will be bound to any other existing Transports, and the FastIR transport will not be installed at all.  This method is equivalent to right-click-install for network INF’s.

Chapter 7 -- Error Codes

This NDIS Miniport Device Driver makes extensive use of the NdisWriteErrorLogEntry(...) function in order to report the cause of any fatal errors during initialization.  When executing on the Windows 95 Operating System, this function will cause a Message Box to be displayed to the Operator indicating the Error Code, as well as logging the error(s) to a disk file:  %system%\NdisLog.Txt.

Error Code 0xC0001388

NDIS_ERROR_CODE_RESOURCE_CONFLICT

This error code indicates that a hardware resource needed by this Driver was already in use by some other Driver.  There are 4 possible resources needed by this Driver:  2 I/O 1 IRQ, and (optionally) 1 DMA.  The particular resource in conflict is indicated by additional values on the line in the NdisLog.Txt file.

0xC0001388, 0x10, SirBase -- 1st I/O resource (ACE UART)

0xC0001388, 0x11, FirBase -- 2nd I/O resource (SCE)

0xC0001388, 0x12, IrqNo -- IRQ resource

0xC0001388, 0x13, DmaCh -- DMA resource (if used)

Error Code 0xC0001389

NDIS_ERROR_CODE_OUT_OF_RESOURCES

This error code indicates that the Driver failed to allocate the memory that it required for operation;  no further information is provided.

Error Code 0xC000138A

NDIS_ERROR_CODE_HARDWARE_FAILURE

This error code indicates that the Driver was able to identify the SMC IrCC at the specified I/O Port locations, but that it was found to be not functioning properly.  Possible results are:

0xC000138A, 0x10, SirBase -- UART failed loopback test

0xC000138A, 0x11, FirBase -- SCE failed PIO data test

0xC000138A, 0x10, SirBase, IrqNo -- UART failed IRQ test

0xC000138A, 0x11, FirBase, IrqNo -- SCE failed IRQ test

0xC000138A, 0x13, FirBase, DmaCh -- SCE failed DMA test

Error Code 0xC000138B

NDIS_ERROR_CODE_ADAPTER_NOT_FOUND

This error code indicates that the Driver was unable to locate an SMC IrCC at the specified I/O Port locations.  Possible results are:

0xC000138B, 0x10, SirBase -- could not find UART @ SirBase

0xC000138B, 0x11, FirBase -- could not find SCE @ FirBase

Error Code 0xC0001391

NDIS_ERROR_CODE_UNSUPPORTED_CONFIGURATION

This error code indicates that the Driver encountered an invalid Configuration Value, either in the Registry when reading the keys corresponding to the entries on the Network-Properties-Advanced sheet, or when obtaining its PnP Hardware Resource Assignments.  Possible results are:

0xC0001391, 0x00, Value -- Reg. Key for RxSel

Must be [0,3]

0xC0001391, 0x01, Value -- Reg. Key for MinTurnTime

Must be 500/1000/5000 (units in Sec)

0xC0001391, 0x10, Value -- PnP value for SirBase, 1st I/O value

Must be [0x0100,0x0ff8], 8-Aligned

0xC0001391, 0x11, Value -- PnP value for FirBase, 2nd I/O value

Must be [0x0100,0x0ff8], 8-Aligned

0xC0001391, 0x12, Value -- PnP value for IrqNo

Must be [0,15], != 2

0xC0001391, 0x13, Value -- PnP value for DmaCh

If provided, must be [0,7], != 4

Error Code 0xC0001393

NDIS_ERROR_CODE_MISSING_CONFIGURATION_PARAMETER

This error code indicates that the Driver encountered a missing  Configuration Value, either in the Registry when reading the keys corresponding to the entries on the Network-Properties-Advanced sheet, or when obtaining its PnP Hardware Resource Assignments.  Possible results are:

0xC0001393, 0x00 -- Reg. Key for RxSel

0xC0001393, 0x01 -- Reg. Key for MinTurnTime

0xC0001393, 0x02 -- Reg. Key for SpeedEnable

0xC0001393, 0x10 -- PnP value for SirBase, 1st I/O value

0xC0001393, 0x11 -- PnP value for FirBase, 2nd I/O value

0xC0001393, 0x12 -- PnP value for IrqNo

Chapter 8 -- Known Bugs

This release of the Driver contains no known bugs.